Chapter III

General Criteria for Selecting a Safe Site

- Introduction

The identity of siting options and the selection of the definitive site for the facility must be based on an assessment of the healthcare needs of the population and the characteristics of the existing health network. The choice of site will also be affected by public health policies and any demographic, geographical, sociopolitical or economic criteria set by the institution. Other important considerations are the performance objectives sought for the facility at normal times and during emergencies, the comparative analysis of the natural and technological hazards present at the various potential sites, the estimated cost and technical feasibility of implementing the necessary protection systems, the economic resources available, and the findings of the cost/benefit analysis of the options as illustrated in Flowcharts 2 and 3.

This assessment must cover not only the specific sites but also their surroundings. The way in which natural phenomena affect the surrounding population, the population of reference and the infrastructure must be evaluated, particularly their impact on lifelines and access roads.

Flow chart 2

Site preselection

SITE PRESELECTION HEALTH, SOCIAL, POLITICAL AND ECONOMIC CONSIDERATIONS SITE C SITE A SITE B SELECTION OF DEFINITIVE SITE IDENTITY ● RISK ASSESSMENT ● POTENTIAL COST ASSESSMENT OF THE SITING OPTION ● ASSESSMENT OF TECHNICAL RESTRICTIONS ● ASSESSMENT OF SITE ADVANTAGES ● COST/BENEFIT ANALYSIS IDENTITY ● RISK ASSESSMENT ● POTENTIAL COST ASSESSMENT OF THE SITING OPTION ● ASSESSMENT OF TECHNICAL RESTRICTIONS ● ASSESSMENT OF SITE ADVANTAGES ● COST/BENEFIT ANALYSIS IDENTITY ● RISK ASSESSMENT ● POTENTIAL COST ASSESSMENT OF THE SITING OPTION ● ASSESSMENT OF TECHNICAL RESTRICTIONS ● ASSESSMENT OF SITE ADVANTAGES ● COST/BENEFIT ANALYSIS

Flow chart 3

Site selection

SITE SELECTION TECHNICAL RESTRICTIONS - HEALTHCARE NEEDS - ECONOMIC RESTRICTIONS CHARACTERISTICS OF HEALTH NETWORK SOCIAL AND POLITICAL RESTRICTIONS SOCIAL AND POLITICAL REQUIREMENTS TECHNOLOGICAL HAZARDS NATURAL HAZARDS PERFORMANCE OBJECTIVES IN NORMAL TIMES PERFORMANCE OBJECTIVES DURING EMERGENCIES

- Process for selecting siting options

  - Variables of site selection

It is not the purpose of this handbook to show in detail how to rank the various siting options.  Instead, relevant criteria will be mentioned, such as the key factors that must be taken into account when selecting an adequate and safe location. It is advisable that the Institution define qualitative and quantitative specifications for assessing and comparing each of the siting options. These specifications may be of varying degrees of complexity. What matters is that they facilitate the decision-making process by testing each site’s capacity to meet the desired performance objective. If none of the siting options can meet it, it will be necessary to select a less ambitious performance objective—or continue searching for acceptable siting options.

All the information on local risks that may be needed for choosing the siting options might be limited to existing data found in land-use management plans, local or regional development plans, technical reports, local zoning laws and regulations, or the opinions of experts. Even so, on-site inspection of each of the options and their surroundings should be carried out by the Assessment and Selection Team. If the health facility will be designed to meet a high performance objective, however, detailed studies must be carried out to characterize the prevailing hazards. No site should be selected if any of the detailed information required is lacking.

In selecting the site, moreover, one must consider the proximity to certain industrial facilities (chemical plants, refineries, mining processing plants, etc.), military facilities, landfills, airports, routes used for the transport of hazardous materials, and so on, facilities that—because of their operations, the emission of toxic agents, or the possibility of eventual accidents at normal times or during an emergency—might affect the safety of the contemplated health facility.

An aspect worth considering is the feasibility of having the local regulatory plan modified in such a way that no activities can be carried out in the future that endanger the hospital and its operations.

  - Site selection procedures

The selection of the site involves three stages, each with its own activities or substages.

Stage 1: Compilation of background data

Stage 2: Assessment of siting options

Stage 3: Site selection

2.2.1	Stage 1: Compilation of background data

2.2.1.1	Preliminary studies

When the different siting options have been selected by the Client Institution, it will be necessary to evaluate any background data available concerning each of the options. The team in charge of site selection must determine whether the available data are sufficient or more information is required in order to compare the various options and select the definitive site. Table 10 lists some of the activities that should be carried out at this stage.

#### Table 10 #### Preliminary activities

Selection of professional team (see Chapter V)

##### Definition of protection objectives and expected level of damage

##### Definition of siting options

Delimitation of the boundaries within which the potential sites would be located

Area to be occupied by the facility

Area of influence

Roads

Lifelines

###### Review of local regulatory plans

Preliminary studies

Human settlements and infrastructure in the region

Inhabited area

Services

Roads and available forms of transportation

Review of existing laws and regulations

Review of regional development plans

Review of existing cartography

Review of general information regarding the sites of interest and their surroundings

Review of background data regarding adverse natural phenomena that have taken place in the region, such as landslides or mudslides, strong winds, floods, seismic events or volcanic eruptions

Compilation of preliminary geotechnical data regarding the potential sites

Compilation of information gathered for other projects that have been developed in the area

Opinion of government bodies and NGOs

Opinion of experts

At this stage, the team of specialists participating in the project must estimate the probability of occurrence of the various natural phenomena considered in this handbook. This is necessary in order to define the level of detail of the studies required to characterize the hazards in the sites chosen. In case not enough information is available, or there are doubts regarding that information, the team of specialists must inform the Project Director and Coordination Team and recommend which studies are needed to characterize the prevailing hazards on each site. The level of detail of the studies will also be determined, naturally, by the performance objective chosen for the facility (see Table 8).

      - Studies required for assessing the risks prevalent in each of the siting options

At the beginning of this stage, the team of specialists must evaluate if the information compiled during the preliminary stage is sufficient to preselect the best potential sites for the facility. If it is, the next step is to carry out a closer evaluation of the siting options, as discussed further on in this chapter. If the information required is not available, the team of specialists must carry out all studies necessary for producing the information that will characterize the hazards prevalent in each site. An example of the types of studies required can be found in Annex A.

2.2.2	Stage 2: Assessment of the siting options

2.2.2.1	Processing the data

The information compiled during the preliminary studies, or that obtained later as needed, must be processed in order to characterize the hazards and level of risk at each of the siting options. Flow chart 4 summarizes the main variables that must be quantified in order to determine the natural hazards prevalent at each site.

Flow chart 4

Quantification of the Risk

QUANTIFICATION OF PREVAILING RISKS EARTHQUAKE DIMENSIONS MAGNITUDE DURATION LIKELIHOOD AFFECTED AREA CHARACTERIZATION DESIGN SPECTRUM SEISMIC VERIFICATION RECORDS DIRECT GEOTECHINICAL CONSEQUENCES MITIGATION POTENTIAL SNOW DIMENSIONS MAGNITUDE DURATION LIKELIHOOD AFFECTED AREA CHARACTERIZATION DESIGN LOAD MITIGATION POTENTIAL STRONG WINDS DIMENSIONS MAGNITUDE DURATION LIKELIHOOD AFFECTED AREA CHARACTERIZATION DESIGN SPEED MITIGATION POTENTIAL LANDSLIDES & MUDSLIDES DIMENSIONS MAGNITUDE DURATION LIKELIHOOD AFFECTED AREA CHARACTERIZATION VOLUME HEIGHT SPEED MITIGATION POTENTIAL FLOODS DIMENSIONS MAGNITUDE DURATION LIKELIHOOD AFFECTED AREA CHARACTERIZATION VOLUME HEIGHT SPEED MITIGATION POTENTIAL VOLCANISM DIMENSIONS MAGNITUDE DURATION LIKELIHOOD CHARACTERIZATION VOLUME SPEED MITIGATION POTENTIAL

To obtain the results summarized in the chart above the tasks specified in tables 11a through 11f must be carried out.

#### Table 11a #### Assessment of landslide risk

Assessment of conditions for the occurrence of a landslide

Historical background

Vegetation

Geological conditions

Natural strata

Topographical conditions

Steepness of slopes

Geomechanical conditions

Stratigraphic sections

Strata of low cohesion and low shear-strength soil

Soil and rock degradation

Watercourse-related hazards (destabilization)

Seismic hazard

Human intervention

Assessment of slope stability

Preliminary assessment

Detailed assessment

Dimensions of the risk of landslides

Surface affected and volume displaced

Velocity of the landslide

Safety factors

Likelihood

Production of landslide risk maps (microzoning)

#### Table 11b #### Assessment of mudslide risk

Assessment of conditions for the occurrence of mudslides

Historical background

Meteorological conditions

Vegetation

Geological conditions

Naturally unstable strata

Topographical conditions

Steepness of slopes

Geomechanical conditions

Stratigraphic sections

Low-cohesion soil with low shear strength

Soil and rock degradation

Drainage and permeability

Human intervention

Dimensions of the risk of mudslides

Surface affected and volume of material displaced

Mudslide speed

Likelihood

Production of mudslide risk maps (microzoning)

#### Table 11c #### Assessment of the risk of strong winds

Assessment of conditions for the occurrence of strong winds

Historical background

Meteorological conditions

Topographical conditions

Dimensions of the risk of strong winds

Wind speed

Likelihood

Production of wind speed maps (microzoning)

#### Table 11d #### Assessment of flood risk

Assessment of conditions for the occurrence of floods

Historical background

Meteorological conditions

Existence of watercourses in the area

Topographical conditions (low-lying areas)

Permeability and use of the soil

Risk of flood by tsunami

Human intervention

Identity of critical points – Calculation of hydraulic axes

Identity of critical overflow points during floods

Dimensions of the risk of flooding

Surface affected

Height of flood water level

Flow speed

Likelihood

Production of flood risk maps (microzoning)

#### Table 11e #### Assessment of seismic risk

Characterization of seismogenic conditions

Establishment of frequency-magnitude ratios

Estimation of the maximum probable seismic event

Estimation of seismic risk

Probabilistic seismic hazard analysis

Definition of attenuation factors

Estimation of the duration of strong ground motion

Estimation of the predominant period of ground motion

Deterministic seismic hazard analysis

Dimensions of the seismic risk

Linear response spectrum

Foundation-soil liquefaction potential

Likelihood of massive landslide (See section on landslides)

Likelihood of tsunamis (See section on floods)

Production of seismic risk maps for each of the siting options

#### Table 11f #### Assessment of volcanic risk

Assessment of likelihood of volcanic activity

Likelihood of lateral explosions

Likelihood of being in the path of pyroclastic flows

Likelihood of being in the path of lava flows

Likelihood of massive landslides

Likelihood of mudslides

Likelihood of contamination by gases and ashes

Likelihood of solid and particulate matter emissions

Likelihood of tsunamis

Dimensions of the risk of volcanic activity

Surface affected (area of influence of volcanic action)

Velocity of the flows

Degree of toxicity of gases emitted

Magnitude of related ground motion

Characterization of probable mudslides (See section on mudslides)

Characterization of probable floods due to tsunamis (See section on floods)

Likelihood

Production of volcanic risk maps (microzoning)

2.2.2.2	Technical and economic feasibility of protection systems

In the case of each likely natural hazard, the technical and economic feasibility of implementing overall protection systems for the structure through the execution of peripheral works and other actions must be evaluated.  The following table lists some of these actions.

#### Table 12 #### Actions that can assist in the overall protection of the facility

Strategies for protection against landslides and mudslides

Slope stabilization

Soil stabilization through the use of geotextiles

Knocking down unstable masses

Reforestation

Cleaning natural watercourses, canals

Construction of drainage facilities

Construction of alluvial terraces

Constant monitoring (instrumentation); early warning systems

Strategies for protection against strong winds

Production of technical detailing specifications

Reforestation

Permanent monitoring of meteorological conditions; early warning systems

Strategies for flood protection

Construction of protection barriers at critical points of the watercourse

Construction of gavions [retaining walls made of rocks and chicken wire] along the full length of the watercourse

Cleaning natural watercourses and canals

Construction of drainage facilities

Reassessment and improvement of rainwater collection and drainage

Reinforcement of the structural system

Others

Strategies for seismic protection

Production of technical specifications for seismic-resistant design

Strategies for protection against volcanic activity

# Permanent monitoring and early warning system

2.2.2.3	Impact of hazards on the sites under consideration

In the case of each prevailing hazard, an assessment must be made of its likely impact on the population to be served, as well as on local lifelines, related agencies, and overall access to health services. The likely impact of the phenomenon on the health network of the region—and, where appropriate, of the country—must also be assessed. This assessment should not only consider the network’s infrastructure but also the functional, economic, and political aspects. All too often, while damage to health infrastructure may be manageable from a technical viewpoint, the political impact can be devastating.

2.2.3	Stage 3: Site selection

2.2.3.1	Selection of the best option

The information compiled must be processed in order to select the safest and most convenient site for the facility. Table 13 summarizes the minimum activities required for the selection of the best site.

#### Table 13 #### Site selection

Production and superimposition of hazard data

Assessment of the level of risk at each of the siting options

Characterization of the hazards present at each site

Cost assessment for the overall protection of the structure

Cost assessment for the protection of key services, systems and components

Vulnerability assessment (likely impact of each hazard)

Comparative cost/benefit analysis of the various options

Selection of definitive site for the structure

In some circumstances it is not possible to meet the desired performance objective due to the extreme conditions in which the target population is forced to live. Given the lack of safe locations, the project’s performance standards should guide siting choices in ways such as the following:

- Dividing the functions of the facility in such a way that they are carried out in different locations that are remote from each other.

- Procuring mobile or temporary facilities.

- Producing effective reference systems so that the population can easily be transferred to health facilities in other areas.

These options make it possible to distribute or reduce the risk.  They also increase costs and make operations more complex than might be desired, but may be the only reasonable alternative if the level of risk is too great.

      - Production of document summaries

The information obtained during the preliminary stage through the risk assessments and the process of site selection must be summarized in a document that should include, at the very least, the following content:

- Explanation of the reasons for the choice of site.

- Description of the risks identified in the site.

- Causes of those risks.

- Characterization of the risks.

- Design recommendations for the facility, including the length of time it can remain cut off from basic services (water, electricity, etc.).

- Design and protection recommendations for the area of influence.

- Protection objectives for the health facility.

- Assessment of site safety

Form 3 below should assist the Project Director and the Coordination Team in selecting a safe site for the hospital.

Form 3

- References

ASCE 7-98, Minimum Design Loads for Buildings and Other Structures, American Society of Civil Engineers, 1998.

FEMA 55: Coastal Construction Manual, Federal Emergency Management Agency, Washington, 1996.

Hallent, B., Photogrammetry: Basic Principles and General Survey, McGraw-Hill, 1960.

Key, D., Structures to Withstand Disasters, Thomas Telford, London, 1995.

Kuroiwa, J., Reducción de Desastres: Viviendo en Armonía con la Naturaleza, Lima, 2002.

Ministry of Housing and Urban Development, Planificación de Asentamientos Humanos en Zonas Propensas a Desastres, Chile, 1982.

PAHO/WHO Collaborating Center for Disaster Mitigation in Health Facilities, Bases Metodológicas: Evaluación de Vulnerabilidad Sísmica de Edificaciones Estructuradas con Pórticos de Preformed concrete, Evaluación de Elementos Arquitectónicos y Evaluación de Equipamiento, University of Chile, 2000.

Pan American Health Organization, Manual para la Mitigación de Desastres Naturales en Sistemas Rurales de Agua Potable, 2001.

Pan American Health Organization, Natural Disaster Mitigation in Drinking Water and Sewerage Systems: Guidelines for Vulnerability Analysis, 2000.

Pan-American Health Organization, Disaster Mitigation for Health Facilities: Guidelines for Vulnerability Appraisal and Reduction in the Caribbean, 2000.

Simiu, E., Wind Effects on Structures: an Introduction to Wind Engineering, John Wiley & Sons, 1978.

Taype, V., “Aplicación de Mapas Geodinámicos en la Prevención de Desastres Naturales”, Fourth National Simposium on the Prevention and Mitigation of Natural Disasters, Lima, CISMID, 1990.